RTL_EXERCISE_1 BOUND FLASHER

Author; Đỗ Nguyễn Đạt. Date; 2021/06/26. Version; 1.2.

Contents

1. Interface	2

2. Functional implementation.	3

3. Internal implementation.	5

3.1. Overall.	5

3.2. State Machine	6

4. History	7

# 1. Interface

LED bound_flasher clk 16 flick rst_n

Figure 1: the figure of Bound Flasher System

| Signal | Width | In/Out | Description |
| --- | --- | --- | --- |
| clk | 1 | In | Generated posedge or negedge clock when it change from low to high or reverse. |
| rst_n | 1 | In | Turned off 16 LEDs when it is activated LOW. |
| flick | 1 | In | Control LED’s state. |
| LED | 16 | Out | Display led’s state (ON or OFF). |

Table 1: Description of signals in Bound Flasher

# 2. Functional implementation.

- Implement a 16-bits LEDs system

- System’s Operation base on three input signal

  - Reset

  - Clock

  - Flick

- The system specification

- Clock signal is provided for system inspire of function status. The function operate state’s transition at positive edge of the clock signal.

- Reset signal:

- LOW-ACTIVE Reset = 0: System is restarted to Initial State.

- HIGH-ACTIVE Reset = 1: System is started with initial state.

- Flick signal: special input for controlling state transfer.

- At the initial state, all lamps are OFF. If flick signal is ACTIVE, the flasher start operating:

- The lamps are turned ON gradually from LEDs [0] to LEDs [5].

- The LEDSs are turned OFF gradually from LEDs [5] to LEDs [0].

- The LEDSs are turned ON gradually from LEDs [0] to LEDs [10].

- The LEDSs are turned OFF gradually from LEDs [10] to LEDs [5].

- The LEDSs are turned ON gradually from LEDs [5] to LEDs [15].

- Finally, the LEDs s are turned OFF gradually from LEDSs [15] to LEDSs [0], return to initial state.

- Additional condition: At each kickback point (LEDs [5] and LEDs [10]), if flick signal is ACTIVE, the LEDs will turn off gradually again to the LED[0] of the previous state and then continue operation as above. For simple, kickback point is considered only when the LEDs s are turned OFF gradually, except the first state.

- Some insulations:

- When flick = 0 at kickback points

- When flick = 1 at kickback points (lamp[10])

# 3. Internal implementation.

## 3.1. Overall.

Figure 3.1: Block diagram of Bound Flasher

Block; Meaning. State-FlipFlop; Control state and the status of the LEDs, triggered by reset or clock. Compare control logic combination; Compare condition of flick and boundary LEDs to update the next state and status of the LEDs..

Table 3.1: Block diagram of Bound Flasher Description

## 3.2. State Machine

Figure 3.2: State Machine of Bound Flasher

| Variable name | Meaning |
| --- | --- |
| rst_n | If reset = 0(LOW), the system will be reset and return to initial state. |
| flick | The control signal to start the system and return to previous state. |
| next_led | The next status of led. For example, next_led = 16’d63, it’s mean we consider the array leds from 0 to 5. |

Table 3.2: variable name of State machine

| State Machine | Meaning |
| --- | --- |
| STOP | At the beginning, all leds are OFF. If flick = 1, system change to next state. |
| UP6 | LEDs are turned on gradually from LED[0] to LED[5]. |
| DOWN6 | LEDs are turned off gradually from LED[5] to LED[0]. |
| UP11 | LEDs are turned on gradually from LED[0] to LED[10]. If flick = 1 at LED[5], LEDs turn off gradually to LED[0]. |
| DOWN11 | LEDs are turned off gradually from LED[10] to LED[0]. |
| DOWNEXTRA | LEDs are turned off gradually from LED[10] to LED[5]. If flick = 1 at LED[4], LEDs turn off gradually to LED[0]. |
| UP16 | LEDs are turned on gradually from LED[0] to LED[15]. If flick = 1 at LED[5] or LED[10], LEDs turn off gradually to LED[0]. |
| DOWN16 | LEDs are turned off gradually from LED[15] to LED[0] and return to initial state. |

Table 3.3: state name of State machine

# 4. History

| Date | Author | Modified part | Description |
| --- | --- | --- | --- |
| 2021/06/18 | Dat | All | New creation |
| 2021/06/26 | Dat | Design spec | Fix almost contents |